Computer Architecture and Organization

Assignment Report — Serial-to-Parallel Converter

GitHub: https://github.com/selenasurucu

https://github.com/emrebeyaziit

https://github.com/efegurayaybek034

yasemin232 (Yasemin)

| # | Student Name and Surname | Student ID | Signature |
| --- | --- | --- | --- |
| 1 | Selena Sürücü | 23040102020 |  |
| 2 | Muharrem Emre Beyazıt | 23040102040 |  |
| 3 | Efe Güray Aybek | 23040102070 |  |
| 4 | Yasemin Özkan | 24040102010 |  |

| Digital Design (25 pts) | Verilog Impl. (25 pts) | Testbench (25 pts) | Report & Pres. (25 pts) |
| --- | --- | --- | --- |

# 1. Introduction

This report presents the design and implementation of an 8-bit Serial-to-Parallel Converter using the Verilog Hardware Description Language. A Serial-to-Parallel converter is a fundamental digital circuit that receives one bit of data per clock cycle through a single wire (serial) and, after a fixed number of clock cycles, presents all received bits simultaneously on multiple wires (parallel).

This type of converter is widely used in modern digital systems, including UART (Universal Asynchronous Receiver Transmitter), SPI (Serial Peripheral Interface), and I2C communication protocols. It bridges the gap between slow serial communication channels and fast parallel data buses.

# 2. Digital Design

## 2.1 Block Diagram

The circuit operates as a synchronous shift register. The architecture consists of three main components: a shift register, a bit counter, and an output register with a valid flag.

┌─────────────────────────────────────────────────────────┐

│                  serial_to_parallel                     │

│                                                         │

│  clk    ──────► ┌───────┐   ┌───────┐                   │

│  rst_n  ──────► │ Shift │──►│ Output│──► parallel_out[7:0]

│  serial_in ───► │ Reg   │   │ Reg   │                   │

│  shift_en ────► │ [7:0] │   └───────┘                   │

│                 └───────┘                               │

│                  ┌─────────┐                            │

│                  │ Bit     │──► data_valid              │

│                  │ Counter │                            │

│                  │ [2:0]   │                            │

│                  └─────────┘                            │

└─────────────────────────────────────────────────────────┘

## 2.2 Signal Description

| Signal | Dir. | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1-bit | System clock (rising-edge triggered) |
| rst_n | Input | 1-bit | Active-low asynchronous reset |
| serial_in | Input | 1-bit | Serial data input, MSB sent first |
| shift_en | Input | 1-bit | Shift enable; 1 = accept serial data |
| parallel_out | Output | 8-bit | Parallel output register |
| data_valid | Output | 1-bit | Pulses HIGH for 1 cycle when word is ready |

## 2.3 Operation Principle

The converter works as follows. When shift_en is asserted (logic 1), on every rising clock edge, a new bit from serial_in is shifted into the least significant bit (LSB) of the internal shift register, while all existing bits shift one position to the left. This means data is received MSB first.

A 3-bit internal counter (bit_count) tracks how many bits have been received. When bit_count reaches DATA_WIDTH-1 (value 7 for 8-bit mode), the current shift register contents are copied to the output register (parallel_out), the data_valid signal pulses HIGH for exactly one clock cycle to notify downstream logic, and the counter resets to zero automatically, ready for the next word.

The reset signal (rst_n) is asynchronous and active-low. Asserting rst_n at any time immediately clears all registers and counters, regardless of the clock. This guarantees a known state on power-up or after an error

# 3. Verilog Implementation

## 3.1 Module Code

The complete Verilog source code (serial_to_parallel.v) is shown below. The module is parameterised by DATA_WIDTH (default 8 bits), allowing it to be reused for 4-bit, 16-bit or 32-bit conversions without changes.

module serial_to_parallel #(

parameter DATA_WIDTH = 8

)(

input  wire                   clk,

input  wire                   rst_n,

input  wire                   serial_in,

input  wire                   shift_en,

output reg  [DATA_WIDTH-1:0]  parallel_out,

output reg                    data_valid

);

reg [$clog2(DATA_WIDTH)-1:0] bit_count;

reg [DATA_WIDTH-1:0]         shift_reg;

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

shift_reg    <= {DATA_WIDTH{1'b0}};

parallel_out <= {DATA_WIDTH{1'b0}};

bit_count    <= {$clog2(DATA_WIDTH){1'b0}};

data_valid   <= 1'b0;

end else begin

data_valid <= 1'b0;

if (shift_en) begin

shift_reg <= {shift_reg[DATA_WIDTH-2:0], serial_in};

bit_count <= bit_count + 1'b1;

if (bit_count == DATA_WIDTH - 1) begin

parallel_out <= {shift_reg[DATA_WIDTH-2:0], serial_in};

data_valid   <= 1'b1;

bit_count    <= {$clog2(DATA_WIDTH){1'b0}};

end

end

end

end

endmodule

## 3.2 Key Design Decisions

- Non-Blocking Assignments (<=): All flip-flop updates inside the always block use non-blocking assignments to correctly model synchronous register behaviour.

- Parameterised Width: DATA_WIDTH is a parameter, making the module reusable. The counter width is automatically sized using $clog2(DATA_WIDTH).

- data_valid Pulse: The valid flag is cleared to 0 at the start of every cycle and only set to 1 on the exact cycle the parallel word is complete. This produces a clean 1-cycle pulse.

- Auto-reset Counter: When bit_count reaches DATA_WIDTH-1, it is explicitly reset to 0 in the same always block, preventing it from overflowing and misaligning subsequent words.

# 4. Verilog Testbench

## 4.1 Testbench Overview

The testbench file (serial_to_parallel_tb.v) uses a 100 MHz clock (10 ns period). It applies the following test scenarios to verify correctness.

- Test 1: 0xA5 (10100101) — alternating bit pattern to verify correct bit ordering.

- Test 2: 0xFF (11111111) — all-ones pattern to detect stuck-at-zero faults.

- Test 3: 0x3C (00111100) — mixed pattern to verify shifting after an all-ones word.

- Test 4: 0x00 (00000000) — all-zeros pattern to detect stuck-at-one faults.

- Test 5: 0x69 (01101001) — pseudo-random pattern for general correctness.

- Test 6: Mid-stream reset recovery — 4 garbage bits are clocked in, then rst_n is asserted mid-stream; after reset the converter must correctly receive 0x55.

- Test 7: shift_en=0 stability — output must remain stable when shifting is disabled.

## 4.2 Simulation Results

All 7 test vectors passed. The simulator output was:

==============================================

Serial-to-Parallel Converter Testbench

DATA_WIDTH = 8 bits

==============================================

[INFO] Reset released. Simulation start.

--- Test 1: 0xA5 (10100101) MSB first ---

[PASS] Test 1: expected=0xA5  got=0xA5

--- Test 2: 0xFF (11111111) ---

[PASS] Test 2: expected=0xFF  got=0xFF

--- Test 3: 0x3C (00111100) ---

[PASS] Test 3: expected=0x3C  got=0x3C

--- Test 4: 0x00 (00000000) ---

[PASS] Test 4: expected=0x00  got=0x00

--- Test 5: 0x69 (01101001) ---

[PASS] Test 5: expected=0x69  got=0x69

--- Test 6: Mid-stream reset, then send 0x55 ---

[INFO] Reset re-asserted and released

[PASS] Test 6: expected=0x55  got=0x55

--- Test 7: shift_en=0, output must stay 0x55 ---

[PASS] Test 7: expected=0x55  got=0x55

==============================================

RESULTS: 7 PASS  /  0 FAIL

==============================================

ALL TESTS PASSED!

## 4.3 GTKWave Waveform Description

The simulation produces a VCD (Value Change Dump) file (serial_to_parallel.vcd) that can be opened with GTKWave to visualise all signals. In the waveform viewer you can verify:

- clk — 10 ns period square wave (100 MHz).

- rst_n — held LOW for 3 cycles, then HIGH for the rest of the simulation.

- shift_en — goes HIGH exactly while a word is being transmitted.

- serial_in — changes on consecutive clock cycles to match each test vector MSB first.

- parallel_out — updates to the correct 8-bit value after the 8th serial bit.

- data_valid — pulses HIGH for exactly 1 clock cycle after each complete word.

# 5. Usage Guide

## 5.1 Compiling and Simulating

Open a terminal in the project folder and run:

# Step 1: Compile design and testbench

iverilog -o sim_out serial_to_parallel_tb.v serial_to_parallel.v

# Step 2: Run simulation (generates .vcd waveform file)

vvp sim_out

# Step 3: Open waveforms in GTKWave

gtkwave serial_to_parallel.vcd

## 5.2 GTKWave Walkthrough

After running gtkwave serial_to_parallel.vcd, follow these steps to display signals.

- In the left panel (SST), expand serial_to_parallel_tb.

- Click on dut to see the module signals.

- Select clk, rst_n, shift_en, serial_in, parallel_out[7:0], data_valid.

- Click Append (or drag signals to the waveform area).

- Press Ctrl+Shift+F (Zoom to Fit) to see the full simulation time.

- Right-click parallel_out and select Data Format > Hexadecimal to view values in hex.

# 6. Conclusion

A fully functional 8-bit Serial-to-Parallel Converter was successfully designed and implemented in Verilog. The design uses a synchronous shift register with a parameterisable data width, an asynchronous active-low reset, a shift-enable control, and a single-cycle data_valid output pulse. All seven testbench scenarios passed, confirming correct operation for normal data reception, all-ones/all-zeros patterns, mid-stream reset recovery, and idle (shift_en=0) behaviour.

The design is compact, synthesisable, and ready for integration into larger digital systems that require serial-to-parallel data conversion, such as UART receivers or SPI slave interfaces.

# References

- Ciletti, M. D. (2011). Advanced Digital Design with the Verilog HDL. Pearson.

- Palnitkar, S. (2003). Verilog HDL: A Guide to Digital Design and Synthesis. Prentice Hall.

- IEEE Standard for Verilog Hardware Description Language, IEEE Std 1364-2005.

- Icarus Verilog Documentation: https://steveicarus.github.io/iverilog/

- GTKWave Documentation: http://gtkwave.sourceforge.net/